### ARDUINO BASED SMART MEDICAL PILL BOX WITH VOICE AND DISPLAY REMINDER FOR PATIENTS

### PROJECT REPORT

Submitted by

GEETHIKA T (VML19EC020)

SAISHNA SHAMEJ (VML19EC040)

SANATH K(VML19EC041)

SHILPA M NAIR (VML19EC045)

in partial fulfillment for the award of the degree of

## BACHELOR OF TECHNOLOGY

in

## ELECTRONICS AND COMMUNICATION ENGINEERING

VIMAL JYOTHI ENGINEERING COLLEGE CHEMPERI

## APJ ABDUL KALAM TECHNOLOGICAL UNIVERSITY

MAY 2023

# DECLARATION

We undersigned hereby declare that the project report “Arduino based smart medical pill box with voice and display reminder for patients”, submitted for partial fulfillment of the requirement for the award of degree of Bachelor of Technology of the Kerala Technical University, Kerala is a bonafide work done by us under the supervision of Dr. Jayesh George M. The submission represents our ideas in our own words and where the ideas or words of others have been included; we have adequately and accurately cited and referenced the original sources. We also declare and we have adhered to ethics of academic honesty and integrity and have not misrepresented or fabricated any data or idea or fact or source in our submission. We understand that any violation of the above will be a cause for disciplinary action by the institute and/or the University and can also evoke penal action from the sources which have thus not been properly cited or from whom proper permission has not been obtained. This report has not been previously formed the basis for the award of any degree, diploma or similar title of any other university.

Place: Chemperi	       GEETHIKA T

SAISHNA SHAMEJ

SANATH K

SHILPA M NAIR

## DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING

### BONAFIDE CERTIFICATE

This is to certify that the report entitled “Arduino based smart medical pill box with voice and display reminder for patients” submitted by GEETHIKA T, SAISHNA SHAMEJ,SANATH K, SHILPA M NAIR to the A P J Abdul Kalam Technological University in partial fulfillment of the requirement for the award of the Degree of Bachelor of Technology in ELECTRONICS AND COMMUNICATION ENGINEERING is a bonafide record of the project work carried out by them under our guidance and supervision. This report in any form has not been submitted to any University or Institute for any purpose.

| Project Guide | Project Coordinator | Head of the department |
| --- | --- | --- |
| Ms. SHIMNA P K | Dr. JAYESH GEORGE M | Dr. D ANTO SAHAYA DHAS |
| Assistant Professor | Associate Professor | Professor and HoD |
| Dept. of ECE | Dept. of ECE | Dept. of ECE |
| Vimal Jyothi Engg college Chemperi, Kannur | Vimal Jyothi Engg college Chemperi, Kannur | Vimal Jyothi Engg college Chemperi, Kannur |

# ACKNOWLEDGEMENT

All glory and honors of God, the Almighty without his grace our project would not have seen the light of success. We wish to acknowledge the following people who are the source of motivation behind it. We would like to thank the management for providing us with the required facilities for the completion of this project. We would like to express our gratitude to Dr. BENNY JOSEPH, Principal for his moral support. We would also like to thank Dr. D ANTO SAHAYA DHAS, Head of the Department, for his valuable suggestions and co-operation. We would like to express our gratitude to our Project Coordinator, Dr. JAYESH GEORGE M, Associate Professor in the Department of ECE, for his valuable support. We wish to extend our heartfelt thanks to our guide Ms. SHIMNA P.K, Assistant professor in the department of ECE, for her guidance, support and cooperation during the course of this project. We would also like to express our gratitude to all the other teaching faculty in the Department of ECE for their support and valuable suggestions. Finally, we would like to thank our parents and friends whose encouragement and advice helped us a lot during the period of time.

ABSTRACT

Nowadays, the use of medicine is rapidly increasing. More than 65% of people forget to take medication, also untimed medicine administration can always show adverse effects on the health of the patients, which creates healthcare issues in our world. Pill reminder and medication adherence is the aim of our project. Several commercial products for automatic pill dispensing or medication reminders are on the market: MedMinder, MedaCube, and Hero. etc. The proposed system is designed to help these patients to take the required medicine in the right proportion at the right time. The basic ideology is integrating the principle of an Alarm clock with Light based slot sensing on a normal pill box. An alternate to the light-based sensing method using capacitive fields is also employed. This medication pill box is focused on patients who frequently take medications or vitamin supplements, or attendants who deal with the more seasoned or patients. Our smart pill box is programmable that enables medical caretakers or clients to determine the pill amount and timing to take pills, and the service times for every day. Our shrewd pills box contains nine separate subboxes. In this manner, medical caretakers or clients can set data for nine distinct pills. At the point when the pill time has been set, the pillbox will remind clients or patients to take pills utilizing sound and light. The warning of pills should be taken will be shown by an android application which is held by the patient. Our shrewd pill box would essentially discharge medical attendants or clients' weight on much of the time preloading pills for patients or clients and overlook the measurements which must be taken.

TABLE OF CONTENTS

| CHAPTER NUMBER | TITLE | PAGE NUMBER |
| --- | --- | --- |
|  | ACKNOWLEDGEMENT | iv |
|  | ABSTRACT | v |
|  | LIST OF FIGURES |  |
| 1 | INTRODUCTION | 1 |
| 2 | LITERATURE REVIEW | 2 |
| 3 | 3.1 BLOCK DIAGRAM 3.2 CIRCUIT DIAGRAM | 3 4 |
| 4 | COMPONENTS 4.1 HARDWARE COMPONENTS 4.1.1 NODE MCU 4.1.2 SERVO MOTOR 4.1.3 LCD DISPLAY 4.1.5 POWER SUPPLY 4.1.6 LED INDICATION 4.2 SOFTWARE COMPONENTS 4.2.1 ARDUINO IDE 4.2.2 MYSQL 4.2.3 PYCHARM | 6 6 13 15 17 20 21 22 22 31 32 |
| 5 | RESULT | 34 |
| 6 | 6.1 CONCLUTION 6.2 FUTURE DEVELOPMENT | 35 36 |
|  | APPENDICES |  |
|  | REFERANCE |  |

LIST OF FIGURES

| Chapter No. | Title | PAGE NUMBER |
| --- | --- | --- |
| 3.1 | BLOCK DIAGRAM | 3 |
| 3.2 | CIRCUIT DIAGRAM | 4 |
| 4.1.1(a) | NODE MCU | 6 |
| 4.1.1(b) | ESP32 WROOM32 DevKit Pinout | 8 |
| 4.1.2 | Servo motor | 14 |
| 4.1.3 | Buzzer | 15 |
| 4.1.3 | Interface to NODE MCU | 16 |
| 4.1.4 (a) | OLED DISPLAY | 17 |
| 4.4.4(b) | OLED DISPLAY PIN DIAGRAM | 18 |
| 4.4.3 | INTERFACE NODE MCU (OLED) | 19 |
| 4.1.5 | POWER SUPPLY | 20 |
| 4.1.6 | LED | 21 |
| 4.2.1.A | Arduino IDE | 22 |
| 4.2.1.(b) | Arduino IDE | 23 |
| 4.2.2 (A) | MENU bar option | 24 |
| 4.2.2 (A) | MENU bar option | 25 |
| 4.2.2.1 (b) | MENU BAR BUTtON | 26 |
| 4.2.2.1 (c) | MENU BAR BUTtON | 27 |
| 4.2.1(D) | TEXT EDITOR | 27 |
| 4.2.1(E) | OUT PUT PANE | 28 |
| 4.2.1(F) | BOARD SELECTION | 29 |
| 4.2.1 (G) | BOARD SELECTION | 29 |
| 4.2.1.(H) | BOARD SELECTION | 30 |
| 4.2.1.(I) | STRUCTURE OF PROGRAM | 31 |
| 5 | RESULT | 35 |

CHAPTER 1

INTRODUCTION

In this decade, people especially old aged individuals have a tendency to forget to consume the prescribed pills or medication at the right time. In certain occasions they completely forget to take the medicines. Most people recognized with chronic diseases are required to take medications over a prolonged period of time in order to stabilize their conditions. If they fail to intake the medicine regularly, it may lead to death. In order to live a better life we need to take medicines regularly as prescribed. From the report of The Centers for Disease Control and Prevention (CDCP), it is estimated that the non-adherence of the medicinal intake causes 30 to 50 percent of chronic disease treatment to fail and over 125,000 deaths per year. It is also found that only 51 percent of patients taking medications for high blood pressure are supposed to continue taking their medication during their long-term treatment. To avoid such kind of deaths one has to take medicines regularly. The project is aimed to overcome the issues mentioned above and to help the people to take medicines at the right time and at regular and prescribed intervals of time. The medicinal combinations for each time of consumption (say morning, afternoon and night) are stored in separate slots. A real time clock is used to monitor the time for the consumption of medicine.  At the right time of medication, the required combinations of pills are collected in a dispatch slot where the pills are ready to be taken by the desired individual once the button is pressed.

    - Objective

The main objective of smart medicine pill box is to remind the elderly people that helps them consume the medicines right on time. The pill box will also give us the information for the re filling of the box. The box will help us to detect the over dosage of the medicine or the wrong dosage that is been used.

CHAPTER 2

LITERATURE REVIEW

According to World Health Organization, over 80% of the people above the age of 60 years are prescribed medicines that are to be administered 2 - 4 times a day. With the increase in Cardio vascular diseases and Diabetes among the peer group regular medicine administration has become a necessity. But among this another 40-60% is having the issues related to forgetting the taking of medicines at right time. The current common techniques used in market for the reminder includes the normal alarm with a pill box. But this does not check for overdose and wrong dosage among the patients. It only uses a clock, which on passage of a set time generates an alarm. Moreover, the timely alerting for the refilling of the pill box to user is also absent resulting often in breaks in the course of therapy. The sensing of slots of the pill box can be done by both Load Sensing methodology and by Light based sensing. The advantages of the slot-based sensing are that individual moment sensing is possible for detecting over dosage problems and incorrect dosage issues. The survey for various modes of sensing the slots has been performed both analytically and practically and comparisons between the modes have been performed. An advanced medicine box monitoring, analysis and control system is proposed in this paper. The latter design is based on a smart and safe medical box that assists patients in taking their pills treatment on time. Two main functionalities characterize this system: safety which assures the wellbeing of the patient and the good functioning of the system by duplicating the electrical components and the security that helps keepingthe medication out of the reach of the children by automatically looking the medical box whenever the patient takes his pills. This system can also be monitored by the patient parents as it will be linked to a phone application. This application will be used to configure the medical box by calculating the weight of each pill, setting the schedule of medical intake, alarming the user of the number of remaining pills, generating alarms whenever the patient does not take the required number of pills or doesn’t take them at all, and so on… This system was implemented and tested by more than 50 patients who were taking several medication types (each one of them takes one medication only) and were using different mobile phone. The overall results were very acceptable with a faulty alarm generation below 3%.

CHAPTER 3

3.1 BLOCK DIAGRAM

Fig 3.1 Block diagram

The Pill Reminder Alarm is powered using 5V supply. When it first boots up, it shows a welcome massage as “Welcome to Circuit Digest”. The LCD screen is set to cycle in three screens. The 1st screen shows massage as “Stay Healthy, Get Well Soon”. The second screen is a help screen which tells to press select push button to select any one time-slot to remind (once/twice/thrice in a day). The time slot is changeable in program and can be configured accordingly. Right now we have fixed this into three durations i.e. 8am, 2pm, and 8pm.

We have divided time slots into three modes. Mode 1 selects to take medicine once/day at 8am when user presses 1st push button. Mode 2 selects to take medicine twice/day at 8am and 8pm when user presses 2nd push button. Mode 3 selects to take medicine thrice/day at 8am, 2pm and 8pm if user presses 3rd push button.

We can also add a feature to snooze the buzzer for 10 minutes (not included in this project). When user selects desired slots by pressing push buttons, the user input is recorded and the time is taken from RTC. When time is matched with selected time slot then the buzzer starts buzzing. User can stop the buzzer by pressing STOP button. The same process continues for the next slot reminder. Complete process is shown in the Video given at the end of this article.

3.2 CIRCUIT DIAGRAM

Fig 3.2 Circuit diagram

Node MCU (ESP 32) is the main control unit of the project, the program code is uploaded to the Node MCU by Arduino IDE. The main power supply to the circuit is provided by a 12V AC adapter. The AC power supply is converted to DC using a Bridge rectifier (Using IN4007 Diodes) circuit, the output have some.AC noise or fluctuations which has been cleared by a capacitor (470 uF).In the circuit we want both 12V and 5V voltages. So the 5V created by using a Voltage Regulator. A led will be always on the circuit when an power is enter to the circuit. A data led will blink when the program is running. The buzzer will alert according to the time set by the user, at that time the compartment which contain the medicine which described at the time will be open for some time, after consume the medicine the compartment will close by the according switch, each compartment have separate switches. If the medicine not consumed the compartment automatically closed and send a message through the telegram observe motors are used to open/close the compartments.

A OLED display is used to show the medicine name at the time, otherwise it shows 'WELCOME'.

CHAPTER 4

COMPONENTS REQUIRED

4.1 HARDWARE DESCRIPTION

1.	NODE MCU

2.	SERVO MOTOR

3.	BUZZER

4.	LCD DISPLAY

5.	POWER SUPPLY

6.	LED INDICATION

4.1.1 NODE MCU

NodeMCU is an open source Lua based firmware for the ESP32 and ESP8266 WiFi SOC from Espressif and uses an on-module flash-based SPIFFS file system. NodeMCU is implemented in C and is layered on the Espressif ESP-IDF.

Fig 4.1.1(a) Node MCU

ESP32-WROOM-32 is a powerful, generic Wi-Fi+BT+BLE MCU module that targets a wide variety of applications, ranging from low-power sensor networks to the most demanding tasks, such as voice encoding, music streaming and MP3 decoding.At the core of this module is the ESP32-D0WDQ6 chip*. The chip embedded is designed to be scalable and adaptive. There are two CPU cores that can be individually controlled, and the CPU clock frequency is adjustable from 80 MHz to 240 MHz. The user may also power off the CPU and make use of the low-power co-processor to constantly monitor the peripherals for changes or crossing of thresholds. ESP32 integrates a rich set of peripherals, ranging from capacitive touch sensors, Hall sensors, SD card interface, Ethernet, high-speed SPI, UART, I2S and I2C.

The integration of Bluetooth, Bluetooth LE and Wi-Fi ensures that a wide range of applications can be targeted, and that the module is future proof: using Wi-Fi allows a large physical range and direct connection to the internet through a Wi-Fi router, while using Bluetooth allows the user to conveniently connect to the phone or broadcast low energy beacons for its detection. The sleep current of the ESP32 chip is less than 5 μA, making it suitable for battery powered and wearable electronics applications. ESP32 supports a data rate of up to 150 Mbps, and 20.5 dBm output power at the antenna to ensure the widest physical range. As such the chip does offer industry-leading specifications and the best performance for electronic integration, range, power consumption, and connectivity. The operating system chosen for ESP32 is freeRTOS with LwIP; TLS 1.2 with hardware acceleration is built in as well. Secure (encrypted) over the air (OTA) upgrade is also supported, so that developers can continually upgrade their products even after their release.

Fig 4.1.1(b)  ESP32 WROOM32 DevKit Pinout

PERIPHERAL FEATURES

- 18 Analog-to-Digital Converter (ADC) channels

- 10 Capacitive sensing GPIOs

- 3 UART interfaces

- 3 SPI interfaces

- 2 I2C interfaces

- 16 PWM output channels

- 2 Digital-to-Analog Converters (DAC)

- 2 I2S interfaces

GPIO(GENERAL PURPOSE INPUT OUTPUT) PINS:

ESP32 Wroom32 DevKit has total 25 GPIOs out of that few pins are Input only Pins

Input Only Pins

- GPIO 34

- GPIO 35

- GPIO 36

- GPIO 39

Not all pins have input pullup, you need external pullup on these pins when using as input pullup.

Pins with internal pull up INPUT_PULLUP

- GPIO14

- GPIO16

- GPIO17

- GPIO18

- GPIO19

- GPIO21

- GPIO22

- GPIO23

Pins without internal pull up

- GPIO13

- GPIO25

- GPIO26

- GPIO27

- GPIO32

- GPIO33

In arduino to use these pins you can simply use common commands

EXAMPLE: To make GPIO22 as input and GPIO23 as output

pinMode(22,INPUT_PULLUP);

pinMode(23,OUTPUT);

digitalWrite(23,HIGH);

Analog Input Pins

Note that only a subset of ADC pins and functions are exposed. First, the supplied drivers expose only ADC1. The board layout of the ESP32-DevKitC only exposes some of the pins. Specifically, the following are exposed: ADC1_CH0 , ADC1_CH3 , ADC1_CH4 , ADC1_CH5 , ADC1_CH6 and ADC1_CH7 .

Capacitive touch GPIOs

The ESP32 has 10 internal capacitive touch sensors. These can sense variations in anything that holds an electrical charge, like the human skin. So they can detect variations induced when touching the GPIOs with a finger. These pins can be easily integrated into capacitive pads, and replace mechanical buttons. The capacitive touch pins can also be used to wake up the ESP32 from deep sleep.

Those internal touch sensors are connected to these GPIOs:

- T0 (GPIO 4)

- T1 (GPIO 0)

- T2 (GPIO 2)

- T3 (GPIO 15)

- T4 (GPIO 13)

- T5 (GPIO 12)

- T6 (GPIO 14)

- T7 (GPIO 27)

- T8 (GPIO 33)

- T9 (GPIO 32)

Digital to Analog Converter (DAC)

There are 2 x 8 bits DAC channels on the ESP32 to convert digital signals into analog  voltage signal outputs. These are the DAC channels:

- DAC1 (GPIO25)

- DAC2 (GPIO26)

RTC GPIOs

There is RTC GPIO support on the ESP32. The GPIOs routed to the RTC low-power subsystem can be used when the ESP32 is in deep sleep. These RTC GPIOs can be used to wake up the ESP32 from deep sleep when the Ultra Low Power (ULP) co-processor is running. The following GPIOs can be used as an external wake up source.

RTC_GPIO0 (GPIO36)

RTC_GPIO3 (GPIO39)

RTC_GPIO4 (GPIO34)

RTC_GPIO5 (GPIO35)

RTC_GPIO6 (GPIO25)

RTC_GPIO7 (GPIO26)

RTC_GPIO8 (GPIO33)

RTC_GPIO9 (GPIO32)

RTC_GPIO10 (GPIO4)

RTC_GPIO11 (GPIO0)

RTC_GPIO12 (GPIO2)

RTC_GPIO13 (GPIO15)

RTC_GPIO14 (GPIO13)

RTC_GPIO15 (GPIO12)

RTC_GPIO16 (GPIO14)

RTC_GPIO17 (GPIO27)

PWM

The ESP32 LED PWM controller has 16 independent channels that can be configured to generate PWM signals with different properties. All pins that can act as outputs can be used as PWM pins (Input only pin GPIOs 34 to 39 can’t generate PWM).

To set a PWM signal, you need to define these parameters in the code:

- Signal’s frequency;

  - Duty cycle;

  - PWM channel;

  - GPIO where you want to output the signal

Serial

ESP32 has three serial ports .First Serial RX0, TX0 is used for programming,

- GPIO3 (U0RXD)

- GPIO1(U0TXD)

Another Serial port is available on

- GPIO16 (U2RXD).

- GIIO17 (U2TXD).

When programming it is named as Serial2.

I2C

When using the ESP32 with the Arduino IDE, you should use the ESP32 I2C default pins (supported by the Wire library):

- GPIO 21 (SDA)

- GPIO 22 (SCL)

SPI

By default, the pin mapping for SPI is:

SPI	MOSI	MISO	  CLK       CS

VSPI	GPIO 23	GPIO 19	GPIO 18	GPIO 5

HSPI	GPIO 13	GPIO 12	GPIO 14	GPIO 15

Interrupts

All GPIOs can be configured as interrupts.

Enable (EN)

Enable (EN) is the 3.3V regulator’s enable pin. It’s pulled up, so connect to ground to disable the 3.3V regulator. This means that you can use this pin connected to a pushbutton to restart your ESP32.

GPIO current drawn

The absolute maximum current drawn per GPIO is source 40mA  and sink 28mAmp according to the “Recommended Operating Conditions” section in the ESP32 datasheet.

4.1.2 SERVO MOTOR

A servo motor is a type of motor that can rotate with great precision. Normally this type of motor consists of a control circuit that provides feedback on the current position of the motor shaft, this feedback allows the servo motors to rotate with great precision. If you want to rotate an object at some specific angles or distance, then you use a servo motor. It is just made up of a simple motor which runs through a servo mechanism

A servo consists of a Motor (DC or AC), a potentiometer, gear assembly, and a controlling circuit. First of all, we use gear assembly to reduce RPM and to increase torque of the motor. Say at initial position of servo motor shaft, the position of the potentiometer knob is such that there is no electrical signal generated at the output port of the potentiometer. Now an electrical signal is given to another input terminal of the error detector amplifier. Now the difference between these two signals, one comes from the potentiometer and another comes from other sources, will be processed in a feedback mechanism and output will be provided in terms of error signal. This error signal acts as the input for motor and motor starts rotating. Now motor shaft is connected with the potentiometer and as the motor rotates so the potentiometer and it will generate a signal. So as the potentiometer’s angular position changes, its output feedback signal changes. After sometime the position of potentiometer reaches at a position that the output of potentiometer is same as external signal provided. At this condition, there will be no output signal from the amplifier to the motor input as there is no difference between external applied signal and the signal generated at potentiometer, and in this situation motor stops rotating.

HARDWARE DEATIALS

Fig 4.1.2 Servomotor

FEATURES

- High efficiency.

- High output power relative to their size.

- More constant torque at higher speed.

- Closed-loop control.

- Quiet operation.

- Highly reliable.

- High ratio of torque to inertia.

- High acceleration

PINOUT

GND -serves as a common ground for the motor and the logic.

5V- is a positive voltage that powers the servo

Control -is an input for the control system.

APPLICATIONS.

- Mostly used for industrial purposes. Important industries such as robotics, pharmaceutics, food services and in-line manufacturing also make use of servos.

- Servos are also most-suited for electrically operated pieces of machinery such as elevators, rudders, walking robots, and operating grippers.

4.1.3 BUZZER

An audio signaling device like a beeper or buzzer may be electromechanical or piezoelectric or mechanical type. The main function of this is to convert the signal from audio to sound. Generally, it is powered through DC voltage and used in timers, alarm devices,

Fig 4.1.3 Buzzer

printers, alarms, computers, etc. Based on the various designs, it can generate different sounds like alarm, music, bell & siren.

The pin configuration of the buzzer is shown below. It includes two pins namely positive

and negative. The positive terminal of this is represented with the ‘+’ symbol or a longer

terminal. This terminal is powered through 6Volts whereas the negative terminal is

represented with the ‘-‘symbol or short terminal and it is connected to the GND terminal.

SPECIFICATIONS

• The frequency range is 3,300Hz

• Operating Temperature ranges from – 20° C to +60°C

• Operating voltage ranges from 3V to 24V DC

• The sound pressure level is 85dBA or 10cm

• The supply current is below 15mA

APPLICATIONS

• Communication Devices

• Electronics used in Automobiles

• Alarm Circuits

• Security Systems

• Timers

• Household Appliances

# INTERFACE TO NODE MCU

# Fig 4.1.3 Interface to NODE MCU

4.1.4 OLED DISPLAY

Fig 4.1.4 (a) OLED Display

This is a 0.96 inch Blue-Yellow OLED display module. The display module can be interfaced with any microcontroller using SPI/IIC protocols. It is having a resolution of 128x64. The package includes display board,display, 4 pin male header presoldered to board.

OLED (Organic Light-Emitting Diode) is a self light-emitting technology composed of a thin, multi-layered organic film placed between an anode and cathode. In contrast to LCD technology, OLED does not require a backlight. OLED possesses high application potential for virtually all types of displays and is regarded as the ultimate technology for the next generation of flat-panel displays.

Specifications

- Viewing angle : greater than 160 degrees

- Supported platforms : for Arduino, 51 series, MSP430 series, STIM32 / 2, SCR chips

- Low power consumption : 0.04W during normal operation

- Support wide voltage : 3.3V-5V DC

- Driver IC : SSD1306

- Communication : IIC, only two I / O ports

- No font : The software takes word module

- Backlight : OLED self light, no backlight

- Interface: VCC: 3.3-5V GND: Ground SCL: Serial Clock SDA: Serial Dat

Pin diagram:

Fig 4.4.4(b)OLED display Pin diagram

Pin Name Description

GND: Ground pin

Power (VCC, VDD, 5V): Single power pin for all the power inputs.

SCK (CLK, SCL):  Performs a common clock signal for both microcontroller/Arduino with OLED.

SDA (MOSI, DI)	: Receives the data from the controlling device/Arduino.

APPLICATION

They are commonly used as a display screen for various electronic projects, as their small form factor and compound functionality help you to save I/O pins on your microcontroller.

4.1.4 INTERFACE TO NODE MCU(OLED)

Fig 4.4.3 Interface to NODE MCU (OLED)

4.1.5 POWER SUPPLY

4.1.5 POWER SUPPLY

An AC adapter or AC/DC adapter is a type of external power supply, often enclosed in a case similar to an AC plug. Other common names include wall wart, power brick, wall charger, and power adapter. Adapters for battery-powered equipment may be described as chargers or rechargers (see also battery charger). AC adapters are used with electrical devices that require power but do not contain internal components to derive the required voltage and power from mains power. The internal circuitry of an external power supply is very similar to the design that would be used for a built-in or internal supply. External power supplies are used both with equipment with no other source of power and with battery-powered equipment, where the supply, when plugged in, can sometimes charge the battery in addition to powering the equipment. Use of an external power supply allows portability of equipment powered either by mains or battery without the added bulk of internal power components, and makes it unnecessary to produce equipment for use only with a specified power source; the same device can be powered from 120 VAC or 230 VAC mains, vehicle or aircraft battery by using a different adapter. Another advantage of these designs can be increased safety; since the hazardous 120 or 240 volt mains power is transformed to a lower, safer voltage at the wall outlet and the appliance that is handled by the user is powered by this lower voltage.

4.1.6 LED

A light-emitting diode (LED) is a two-lead semiconductor light source. It is a p–n junction diode, which emits light when activated. When a suitable voltage is applied to the leads, electrons are able to recombine with electron holes within the device, releasing energy in the form of photons. This effect is called electroluminescence, and the color of the light (corresponding to the energy of the photon) is determined by the energy band gap of the semiconductor

Fig 4.1.6 LED

SOFTWARE DESCRIPTION

4.2.1 ARDUINO IDE

Fig 4.2.1.(A)Arduino IDE

Arduino IDE is an open-source software, designed by Arduino.cc and mainly used for writing, compiling & uploading code to almost all Arduino Modules. It is an official Arduino software, making code compilation too easy that even a common person with no prior technical knowledge can get their feet wet with the learning process. It is available for all operating systems i.e., MAC, Windows, Linux and runs on the Java Platform that comes with inbuilt functions and commands that play a vital role in debugging, editing and compiling the code.

Fig 4.2.1.(b) Arduino IDE

A range of Arduino modules available including Arduino Uno, Arduino Mega, Arduino Leonardo, Arduino Micro and many more. Each of them contains a microcontroller on the board that is actually programmed and accepts the information in the form of code. The main code, also known as a sketch, created on the IDE platform will ultimately generate a Hex File which is then transferred and uploaded in the controller on the board. The IDE environment mainly contains two basic parts: Editor and Compiler where former is used for writing the required code and later is used for compiling and uploading the code into the given Arduino Module. This environment supports both C and C++ languages. The IDE environment is mainly distributed into three sections. Menu Bar, Text Editor, Output Pane. As you download and open the IDE software, it will appear like an image below:

MENU BAR OPTIONS

The bar appearing on the top is called Menu Bar that comes with FIVE different options as follow

File: You can open a new window for writing the code or open an existing one. The following table shows the number of further subdivisions the file option is categorized into.

Fig 4.2.2 (A) MENU bar option

Edit: Used for copying and pasting the code with further modification for font

Sketch: For compiling and programming

Tools: Mainly used for testing projects. The Programmer section in this panel is used for burning a bootloader to the new microcontroller.

Help: In case you are feeling sceptical about software, complete help is available from getting started to troubleshooting.

4.2.2.C MENU BAR BUTTON

The Six Buttons appearing under the Menu tab are connected with the running program as follows.

Fig 4.2.2.1 (b)MENU BAR BUTtON

The checkmark appearing in the circular button is used to verify the code. Click this once you have written your code.

The arrow key will upload and transfer the required code to the Arduino board.

The dotted paper is used for creating a new file.

The upward arrow is reserved for opening an existing Arduino project.

The downward arrow is used to save the current running code.

The button appearing on the top right corner is a Serial Monitor.

A separate pop-up window that acts as an independent terminal and plays a vital role in sending and receiving the Serial Data. You can also go to the Tools panel and select Serial Monitor, or pressing Ctrl + Shift + M all at once will open it instantly. The Serial Monitor will actually help to debug the written Sketches where you can get a hold of how your program is operating. Your Arduino Module should be connected to your computer by USB cable in order to activate the Serial Monitor.

You need to select the baud rate of the Arduino Board you are using right now. For my Arduino Uno Baud Rate is 9600, as you write the following code and click the Serial Monitor, the output will show as the image below.

Fig 4.2.2.1 (c)MENU BAR BUTtON

TEXT EDITOR

The main screen below the Menu bard is known as a simple text editor used for writing the required code.

Fig 4.2.1(D) TEXT EDITOR

OUTPUT PANE

The bottom of the main screen is described as an Output Pane that mainly highlights the compilation status of the running code: the memory used by the code, and errors that occurred in the program. You need to fix those errors before you intend to upload the hex file into your Arduino Module.

Fig 4.2.1(E)OUT PUT PANE

Arduino C language works similar to the regular C language used for any embedded system microcontroller, however, there are some dedicated libraries used for calling and executing specific functions on the board.

ARDUINO LIBRERIES

Libraries are very useful for adding extra functionality into the Arduino Module. There is a list of libraries you can check by clicking the Sketch button in the menu bar and going to Include Library.

As you click the Include Library and Add the respective library it will be on the top of the sketch with a #include sign. Suppose, I Include the EEPROM library, it will appear on the text editor as

#include <EEPROM.h>

Most of the libraries are preinstalled and come with the Arduino software. However, you can also download them from external sources.

Fig 4.2.1(F)ARDUINO LIBRERIES

BOARD SELECTION

In order to upload the sketch, you need to select the relevant board you are using and the ports for that operating system. As you click the Tools on the menu, it will open like the figure below:

Fig 4.2.1 (G) BOARD SELECTION

Just go to the “Board” section and select the board you aim to work on. Similarly, COM1, COM2, COM4, COM5, COM7 or higher are reserved for the serial and USB board. You can look for the USB serial device in the ports section of the Windows Device Manager.

The following figure shows the COM4 that I have used for my project, indicating

Fig 4.2.1.(H) BOARD SELECTION

the Arduino Uno with the COM4 port at the right bottom corner of the screen

After correct selection of both Board and Serial Port, click the verify and then upload button appearing in the upper left corner of the six-button section or you can go to the Sketch section and press verify/compile and then upload. The sketch is written in the text editor and is then saved with the file extension.info. It is important to note that the recent Arduino Modules will reset automatically as you compile and press the upload button the IDE software, however, the older versions may require the physical reset on the board. Once you upload the code, TX and RX LEDs will blink on the board, indicating the desired program is running successfully.

STRUCTURE OF PROGRAM

The structure of Arduino program is pretty simple. Arduino programs have a minimum of 2 blocks, Preparation & Execution. Each block has a set of statements enclosed in curly braces. Here, setup () is the preparation block and loop () are an execution block.

Fig 4.2.1.(I) STRUCTURE OF PROGRAM

The setup function is the first to execute when the program is executed, and this function is called only once. The setup function is used to initialize the pin modes and start serial communication. This function has to be included even if there are no statements to execute.

After the setup () function is executed, the execution block runs next. The execution block hosts statements like reading inputs, triggering outputs, checking conditions etc. In the above example loop () function is a part of execution block. As the name suggests, the loop () function executes the set of statements (enclosed in curly braces) repeatedly.

4.2.2 MySQL

MySQL is the world's most popular open source database software, with over 100 million copies of its software downloaded or distributed throughout its history. With its superior speed, reliability, and ease of use, MySQL has become the preferred choice for Web, Web 2.0, SaaS, ISV, Telecom companies and forward-thinking corporate IT Managers because it eliminates the major problems associated with downtime, maintenance and administration for modern, online applications. Many of the world's largest and fastest-growing organizations use MySQL to save time and money powering their high-volume Web sites, critical business systems, and packaged software — including industry leaders such as Yahoo!, Alcatel- Lucent, Google, Nokia, Wikipedia, and Booking.com. The flagship MySQL offering is MySQL Enterprise, a comprehensive set of production tested software, proactive monitoring tools, and premium support services available in an affordable annual subscription.

FEATURES OF MYSQL

• Ease of use—Go from download to complete installation in less than 15 minutes.

• Low TCO—Deploy MySQL for mission-critical applications with significant cost savings over Microsoft SQL Server.

Scalability and performance Meet the scalability and performance requirements of the most trafficked web sites and the most demanding applications.

• Production support—Oracle Premier Support helps lower the total cost and risk of owning you’re mySQL solutions

• MySQL is a database management system.

• MySQL databases are relational.

• MySQL software is Open Source.

• The MySQL Database Server is very fast, reliable, scalable, and easy to use.

4.2.3 Pycharm

PyCharm is the most popular IDE used for Python scripting language. It is developed by the Czech company jetBrains.it provides code analysis, a graphical debugger, an integrated unit tester, integration with version control systems and supports web development with Django.

PYCHARM FEATURES:

PyCharm offers some of the best features to its users and developers in the following aspects:

- Code completion and inspection Advanced debugging Support for web programming and frameworks such as Django and Flask Features of PyCharm Besides, a developer will find PyCharm comfortable to work with because of the features mentioned below:

- Code Completion PyCharm enables smoother code completion whether it is for built in or for an external package

- SQLAlchemy as Debugger You can set a breakpoint, pause in the debugger and can see the SQL representation of the user expression for SQL Language code.

Git Visualization in Editor When coding in Python, queries are normal for a developer. You can check the last commit easily in PyCharm as it has the blue sections that can define the difference between the last commit and the current one.Code Coverage in Editor You can run .py files outside PyCharm Editor as well marking it as code coverage details elsewhere in the project tree, in the summary section etc.

CHAPTER 5

RESULT

FIG 5 RESULT

- All the objectives had been fulfilled successfully and thus we are able to design a project that would remind the patients to have their medicine on  time

- By using an Arduino-based smart pill box with voice and display reminder, patients can be reminded to take their medication on time and in the correct dosage

- The device can be programmed to provide customized reminders based on the patients medication schedule, and the voice feature can provide additional auditory cues to ensure the patient does not forget to take their medication

CHAPTER 6

6.1 CONCLUSION

The proposed system consists of a safety-related medicalbox that can alert the patient, via a phone application, about the time to take his medication and if the correct medicine dose was taken, the number of remaining pills in the box and auto-locks the box to keep the medicine out of reach of children. This system is, for sure, not the first one that helps monitoring and assisting patients. Several previous published works have proposed such systems as the design of smart homes fully equipped by sensors, the monitoring of patients’ walk and fall, the telemedicine systems that monitors patients from home, Added to that, as mobile phones are playing an important role in today’s life, the connection of such medical systems to mobile devices is increasing dramatically due to the ease monitoring and alarm generation. As a human-related system is proposed, the safety and reliability issues are to be considered. These features must be provided mainly when transmitting data whether by making sure of the correct data delivery or the exact receiving part.

6.2 FUTURE DEVELOPMENTS

- The pill box contain medical equipment's  like thermometer, glucometer etc, which can be used to measure the patients present heath condition under doctor's supervision.

- At present the details are provided manually by the patient ,in the future the box will automatically detect the readings from the equipment  and upload to the server.

- At present the patients only connected with one doctor or pharmacist, later we would like to create a hub that consists of doctors and experienced persons to provide the medical assistance at anytime.

APPENDICES

REFERENCES

1. Brown, Marie T, and Jennifer K Bussell. “Medication Adherence: WHO Cares?” Mayo Clinic Proceedings, Mayo Foundation for Medical Education and Research, Apr. 2011, www.ncbi.nlm.nih.gov/pmc/articles/PMC3068890/.

2. Heath, Sara. “5 Ways to Improve Medication Adherence in Chronic Care Patients.” Patient Engagement HIT, 22 Aug. 2017, patientengagementhit.com/news/5-ways-to-improvemedication-adherence-in-chronic-care-patients.

3. Jabeena, A, et al. “Automatic Pill Reminder For Easy Supervision.” Https://Arxiv.org/, Cornell University, 2018,arxiv.org/ftp/arxiv/papers/1711/1711.09739.pdf.

4. Jadhav, Rushikesh, et al. “Intelligent Pillbox for Monitering the Health Using IoT Concepts.” Https://Www.irjet.net/, International Research Journal of Engineering and Technology (IRJET), July 2020, www.irjet.net/archives/V7/i7/IRJET-V7I7707.pdf.

5. Leung, Audrey. “Smart Auto-Dispensing Pillbox.” Arduino — Project Hub, Project Hub, Dec. 2015, create.arduino.cc/projecthub/audalogy/smart-auto-dispensing-pillbox-b1c089.